### Monitor PZP Vibratory Probe

Product Overview:
The Monitor PZP Vibratory Probe is a compact, highly sensitive level sensor used for detecting dry materials in hoppers and silos. It uses piezoelectric technology to detect the presence or absence of material by monitoring the damping effect on a vibrating blade.

3 Key Features:

- Compact and sensitive design

- No calibration required

- Piezoelectric technology for high reliability

Technical Specifications:

- Sensing Element: Vibrating probe (rod or blade)

- Power Supply: 24 VDC or 90–260 VAC

- Output: SPDT or DPDT relay

- Process Temp: -20°C to +150°C

- Sensitivity: Adjustable for light or dense media

Applications & Industry Use-Cases:

- Plastic granules, powdered chemicals, wood pellets

- Batching systems, mixing vessels, or feed hoppers

- Food-grade and pharmaceutical-grade options available

Media Compatibility:

- Fine powders, granules, and dry bulk solids

Capabilities:

- Detects material contact through vibration damping

- Maintenance-free operation with no moving parts

- Highly responsive and repeatable performance

Optional Accessories:

- Sanitary fittings

- Remote electronics for high-temperature zones

- Probe extensions for deep vessels