## Valbia Pneumatic Actuator Series

### 1. Product Overview

Valbia pneumatic actuators are high-performance, rack-and-pinion quarter-turn actuators engineered for industrial automation of ball, butterfly, and plug valves. Built with compact, corrosion-resistant construction, they offer reliable actuation with a variety of rotational and operating modes, backed by a patented design that enhances longevity and reduces leak paths.

### 2. Series & Variant Types

| Series / Variant | Rotation | Material | Functionality |
| --- | --- | --- | --- |
| Series 82 | 0°–90° | Hard-anodized aluminum | Double-acting or spring-return (single-acting) |
| Series 83 | 0°–180° | Hard-anodized aluminum | Double-acting with adjustable travel and cam settings |
| Series 84 | 0°–90° | Stainless steel | DA/SR versions for harsh or corrosive environments |

Special versions include nickel-plated and PTFE-coated finishes for extra protection, as well as temperature-capable designs down to –40 °C or up to +150 °C.

### 3. 3 Key Features

- Choice of 0–90° or 0–180° rotation via three series

- Robust alloy construction with anti-blowout rack design

- Custom coatings and seals for extreme environments

### 4. Technical Specifications

- Design: Twin rack-and-pinion for balanced torque and compact form

- Materials & Protections:

  - Series 82/83: Extruded hard-anodized aluminum bodies with nickel-plated steel pinions

  - Series 84: Stainless steel bodies for enhanced corrosion resistance

- Seals: Standard FKM (Viton); optional silicone or low-temp variations

- Rotation Adjustment: ±5° via stainless steel cams and bolts

- Mounting & Interfaces:

  - Output shaft: ISO 5211

  - Solenoid interface: Namur pattern

- Pressure & Temp Ratings:

  - Max Pressure: ~8 bar (116 psi)

  - Operating Temp Range: –40 °C to +150 °C (depending on seals and coatings)

- Standards: ATEX Zone 1/2 certification for explosive environments; PED/CE compliance

### 5. Applications & Industry Use-Cases

- Industrial process actuation in chemical, food, and water sectors

- Automation of valves in corrosive, high-temp, or cryogenic environments

- High-cycle and safety-critical operations using modulating or failsafe versions

- Marine and offshore valve automation requiring rugged design

### 6. Capabilities

- Highly durable, low-maintenance rack-and-pinion mechanism

- Precision control with tight end-position adjustment

- Uniform torque output for consistent valve operation

- Multiple material and finish combinations to match process conditions

- Safe differential mounting/interface for accessories like limit switches or solenoids

### 7. Optional Accessories & Variants

- Spring-Return (SR) Versions: Fail-safe function for valve positioning under pressure loss

- Double-Acting (DA) Versions: Controlled open and close via air pressure in both directions

- Special Finishes: Nickel plating or PTFE coatings for maximal corrosion resistance

- High/Low Temp Options: Silicone, Viton, or NBR seals to suit extreme thermal environments

- Accessories: Solenoid valve mounts, limit switches, manual overrides, and positioners