### Valpres LOCPOWER

Product Overview:
The Valpres LOCPOWER series represents a new generation of robust, reliable electric actuators designed for precision valve control in industrial automation. Tailored for ball and butterfly valves, this smart actuator system offers advanced positioning, customizable torque outputs, and user-friendly local control. Engineered for efficiency and safety, LOCPOWER actuators are built to handle a wide range of industrial requirements with maximum reliability.

3 Key Features:

- Torque Range: 15 Nm to 4000 Nm

- Multi-voltage compatibility

- IP68-rated for heavy-duty environments

Product Variants & Details:

| Model / Variant | Torque Output | Voltage Options | Enclosure | Control Options |
| --- | --- | --- | --- | --- |
| LOCPOWER 15 – 60 | 15–60 Nm | 24V AC/DC, 100–240V AC | IP68 | ON/OFF, Modulating, Fail Safe |
| LOCPOWER 100 – 400 | 100–400 Nm | 24V AC/DC, 100–240V AC | IP68 | ON/OFF, Modulating, Fail Safe |
| LOCPOWER 800 – 1500 | 800–1500 Nm | 230V AC, 400V AC | IP68 | ON/OFF, Modulating, Remote Control |
| LOCPOWER 3000 – 4000 | 3000–4000 Nm | 400V AC | IP68 | ON/OFF, Remote Positioning, BUS Protocols |

Technical Specifications:

- Ambient Temperature: -20°C to +70°C

- Duty Cycle: S4 – 50%

- Control Interface: Digital, Local Display, or BUS Communication

- Actuation Time: Adjustable per model

- Mechanical Position Indicator: Included

- Mounting: ISO 5211 flange compatibility

Applications:

- Oil & Gas pipelines

- Water treatment plants

- HVAC systems

- Food and beverage processing

- Chemical dosing systems

Certifications & Standards:

- CE Marked

- IP68 Waterproof Rating

- ISO 9001 Compliant

- Optional ATEX variants on request

Accessories & Options:

- Manual Override

- Heater & Thermostat

- Analog Position Feedback (4–20 mA)

- Digital Communication: Modbus, Profibus, others on request

- Fail-Safe Battery Backup System